ISO

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

---------------------------------------------------------------------------------------

ISO/IEC  JTC1/SC2/WG2

Universal Multiple-Octet Coded Character Set  (UCS)

--------------------------------------------------------------------------------

ISO/IEC JTC1/SC2/WG2 N ...

Date:  1998-11-11

TITLE:		Editorial Corrigendum on Zones and related features of 10646.-1

SOURCE:	Bruce Paterson, project editor

STATUS:	Response to Resolution M35.10

ACTION:	For review and confirmation by WG2

DISTRIBUTION:	JTC1/SC2/WG2

This paper provides a draft Editorial Corrigendum for ISO/IEC 10646-1 to remove the four zones that were defined in the First Edition, namely the A-zone, I-zone, O-zone, and R-zone, in reponse to Resolution M35.10 taken at the London meeting in September 1998.  The S-zone, which is reserved for UTF-16, and the private use zone, which is never used for other purposes, are retained.

This paper also proposes the renaming of a few contiguous blocks of characters as single blocks, where the characters all belong to the same script and the block names merely duplicate the existing collection names.  Such a change was recently made for the HEBREW blocks in Corrigendum no.2.

1.  Zones.

The required amendments to clauses 8, 10, and 11, and to Figures 2, 3 and 4, are shown on the following pages.  Some material has been swapped between clauses 10 and 11 so that all the requirements relating to private use characters are now brought together in a single new clause 10.

This draft takes WG2 N 1796 as the basis for the current text of 10646-1, but Figures 3 and 4 now include all Amendments and drafts up to PDAM.31.

2.  Blocks

The following changes are proposed in Annex A.2 “Blocks in the BMP”.

Replace

- BASIC GREEK			0370-03CF

- GREEK SYMBOLS AND COPTIC	03D0-03FF	by  GREEK		0370-03FF

Replace

- BASIC ARABIC			0600-065F

- ARABIC EXTENDED 		0660-06FF	by  ARABIC		0600-06FF

Replace

- GEORGIAN EXTENDED		10A0-10CF

- BASIC GEORGIAN			10D0-10FF 	by  GEORGIAN	10A0-10FF

## 8  The Basic Multilingual Plane

[existing text]

Plane 00 of Group 00 shall be the Basic Multilingual Plane (BMP). The BMP can be used as a two-octet coded character set in which case it shall be called UCS-2 (see 14.1).

The Basic Multilingual Plane shall be divided into five  zones:

A-zone: 	code positions 0000 0000 to 0000 4DFF

I-zone: 	code positions 0000 4E00 to 0000 9FFF

O-zone:	code positions 0000 A000 to 0000 D7FF

S-zone:	code positions 0000 D800 to 0000 DFFF

R-zone:	code positions 0000 E000 to 0000 FFFD

|  | 00 | FF |
| --- | --- | --- |
| 00 | A-zone (19903 positions) |  |
| 4E | I-zone (20992 positions) |  |
| A0 | O-zone (14336 positions) |  |
| D8 | S-zone (2048 positions) |  |
| E0 | R-zone (8190 positions) |  |

Code positions 0000 0000 to 0000 001F in the BMP are reserved for control characters, and code position 0000 007F is reserved for the character DELETE (see clause 16). Code positions 0000 0080 to 0000 009F are reserved for control characters.

In the Basic Multilingual Plane, the A-zone is used for alphabetic and syllabic scripts together with various symbols. The I-zone is used for Chinese/Japanese/Korean (CJK) unified ideographs (unified East Asian ideographs). The O-zone is used for Korean Hangul syllables, and for various other scripts.  The S-zone is reserved for the use of UTF-16 (see Annex Q).  The R-zone shall be used for the restricted use zone in the BMP which contains private use characters, presentation forms, and compatibility characters (see clause 10) .

## 8  The Basic Multilingual Plane

[proposed amended text;  new text is shown underlined, deleted text is not shown]

Plane 00 of Group 00 shall be the Basic Multilingual Plane (BMP). The BMP can be used as a two-octet coded character set in which case it shall be called UCS-2 (see 14.1).

Code positions 0000 0000 to 0000 001F in the BMP are reserved for control characters, and code position 0000 007F is reserved for the character DELETE (see clause 16). Code positions 0000 0080 to 0000 009F are reserved for control characters.

Code positions 0000 D800 to 0000 DFFF are reserved for the use of UTF-16 (see Annex Q).  These positions are known as the S-zone.

Code positions 0000 E000 to 0000 F8FF are reserved for private use (see clause 10).  These positions are known as the private use zone.

Code postions FFFE and FFFF are reserved.

[existing text]

## 10  The restricted use zone

Sets of graphic characters that are used in particular ways are provided in the restricted use zone. These sets include:

a) Private use characters,

b) Presentation forms of characters,

c) Compatibility characters.

### 10.1  Private use characters

Private use characters are not restrained in any way by ISO/IEC 10646. Private use characters can be used to provide user-defined characters. For example, this is a common requirement for users of ideographic scripts.

NOTE 1 - For meaningful interchange of private use characters, an agreement, independent of ISO/IEC 10646, is necessary between sender and recipient.

Private use characters can be used for dynamically-redefinable characters applications.

NOTE 2 - For meaningful interchange of  dynamically-redifinable characters, an agreement, independent of ISO/IEC 10646 is necessary between sender and recipient. ISO/IEC 10646 does not specify the techniques for defining or setting up dynamically-redefinable characters.

### 10.2  Presentation forms of characters

Each presentation form of character provides an alternative form, for use in a particular context, to the nominal form of the character or sequence of characters from the other zones of graphic characters. The transformation from the nominal form to the presentation forms may involve substitution, superimposition, or combination.

The rules for the superimposition, choice of differently shaped characters, or combination into ligatures, or conjuncts  which are often of extreme complexity  are not specified in ISO/IEC 10646.

In general, presentation forms are not intended to be used as a substitute for the nominal forms of the graphic characters specified elsewhere within this coded character set. However, specific applications may encode these presentation forms instead of the nominal forms for specific reasons among which is compatibility with existing devices. The rules for searching, sorting, and other processing operations on presentation forms are outside the scope of ISO/IEC 10646.

[proposed amended text]

## 10  Private use groups, planes, and zones

### 10.1  Private use characters

Private use characters are not restrained in any way by ISO/IEC 10646. Private use characters can be used to provide user-defined characters. For example, this is a common requirement for users of ideographic scripts.

NOTE 1 - For meaningful interchange of private use characters, an agreement, independent of ISO/IEC 10646, is necessary between sender and recipient.

Private use characters can be used for dynamically-redefinable character applications.

NOTE 2 - For meaningful interchange of dynamically-redefinable characters, an agreement, independent of ISO/IEC 10646 is necessary between sender and recipient. ISO/IEC 10646 does not specify the techniques for defining or setting up dynamically-redefinable characters.

### 10.2  Code positions for private use characters

The code positions of the 32 groups from Group 60 to Group 7F shall be for private use.

The code positions of Plane 0F and Plane 10, and of the 32 planes from Plane E0 to Plane FF, of Group 00 shall be for private use.

The 6400 code positions E000 to F8FF of the Basic Multilingual Plane shall be for private use.

The contents of these code positions are not specified in ISO/IEC 10646 (see 10.1).

## 11  Sets of characters with particular uses

Sets of graphic characters that are used in particular ways are provided in ISO/IEC 10646. These sets include:

a) Presentation forms of characters,

b) Compatibility characters.

### 11.1  Presentation forms of characters

Each presentation form of a character provides an alternative form, for use in a particular context, to the

......................   [unchanged from 10.2]   ..................

...

...

searching, sorting, and other processing operations on presentation forms are outside the scope of ISO/IEC 10646.

Within the BMP these characters are mostly allocated to positions in rows FB to FF.

[existing text, continued]

### 10.3  Compatibility characters

Compatibility characters are included in ISO/IEC 10646 primarily for compatibility with existing coded character sets to allow two-way code conversion without loss of information.

## 11  Private use groups, planes, and zones

The code positions of the 32 groups from Group 60 to Group 7F shall be for private use.

The code positions of Plane 0F and Plane 10, and of the 32 planes from Plane E0 to Plane FF, of Group 00 shall be for private use.

The 6400 code positions E000 to F8FF of the Basic Multilingual Plane shall be for private use.

The contents of these code positions are not specified in ISO/IEC 10646 (see 10.1).

[proposed amended text, continued]

### 11.2  Compatibility characters

Compatibility characters are included in ISO/IEC 10646 primarily for compatibility with existing coded character sets to allow two-way code conversion without loss of information.

Within the BMP many of these characters are allocated to positions within rows F9, FA, FE, and FF, and within rows 31 and 33.  Some compatibility characters are also allocated within other rows.

Editor’s note:  It would be preferable to move this new clause 11 to a later position in the text, for example following:

- 19  Block names

and before:

- 20  Characters in bi-directional context

- 21  Special characters

- 22  Order of characters

The intervening clauses 12 to 19 would then become 11 to 18.

Supplementary planes

Cell-octet

00	80	FF

Row-

octet

FF

80

E0

Private Use

80	planes

D8	S-zone

E0	Private use zone

01

F9

FF	00

Basic Multilingual Plane			Plane-octet

Labels “S-zone” and “Private use zone” are specified in clause 8.

## Figure 2 - Group 00 of the Universal Multiple-Octet Coded Character Set

Row-octet

| 00 .. .. .. .. .. 33 |  | Rows 00 to 33 (see Figure 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 .. 4D |  | CJK Unified Ideographs Extension A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4E .. .. .. .. .. .. 9F |  | CJK Unified Ideographs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A0.. A3 |  |  |  | Yi Syllables |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A4 |  |  |  |  |  |  |  |  |  |  | Yi Radicals |  |  |  |  |  |  |
| A5.. AB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AC .. .. .. .. D7 |  |  |  | Hangul Syllables |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D8.. DF |  |  |  | S-zone (for use in UTF-16 only) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0 .. .. F8 |  |  |  | Private Use Zone |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F9 FA |  |  |  | CJK Compatibility Ideographs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FB |  |  |  | Alphabetic Presentation Forms |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FC FD |  |  |  | Arabic Presentation Forms-A |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FE |  |  |  | Comb. Half M’ks |  | CJK Compat. F’ms |  | Small Form Vars. |  |  | Arabic Presentation Forms-B |  |  |  |  |  |
| FF |  |  |  | Halfwidth And Fullwidth Forms |  |  |  |  |  |  |  |  |  | Specials |  |  |  |
|  |  |  |  |  |  | = not graphic characters |  |  |  |  |  |  | = reserved for future standardization |  |  |  |  |

NOTE:  Vertical boundaries within rows are indicated in approximate positions only.

Figure 3 - Overview of the Basic Multilingual Plane

Row-octet

| 00 |  | Basic Latin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Latin-1 Supplement |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | Latin Extended-A |  |  |  |  |  |  |  |  |  |  |  |  |  | Latin Extended-B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02 | Latin Extended-B |  |  |  |  |  | IPA (Int. Phon. Alph.) Extensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Spacing Modifier Letters |  |  |  |  |  |  |
| 03 | Combining Diacritical Marks |  |  |  |  |  |  |  |  | Basic Greek |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Greek Symbols and Coptic andadn |  |  |  |
| 04 | Cyrillic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 05 |  |  |  | Armenian |  |  |  |  |  |  |  |  |  |  |  |  | Hebrew |  |  |  |  |  |  |  |  |  |  |  |  |
| 06 | Basic Arabic |  |  |  |  |  |  |  | Arabic Extended |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 07 | Syriac |  |  |  |  |  |  |  |  |  |  |  |  |  | Thaana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 09 | Devanagari |  |  |  |  |  |  |  |  |  |  |  |  |  | Bengali |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0A | Gurmukhi |  |  |  |  |  |  |  |  |  |  |  |  |  | Gujarati |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0B | Oriya |  |  |  |  |  |  |  |  |  |  |  |  |  | Tamil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0C | Telugu |  |  |  |  |  |  |  |  |  |  |  |  |  | Kannada |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0D | Malayalam |  |  |  |  |  |  |  |  |  |  |  |  |  | Sinhala |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0E | Thai |  |  |  |  |  |  |  |  |  |  |  |  |  | Lao |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0F | Tibetan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Myanmar |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Georgian |  |  |  |  |  |  |  |  |  |  |
| 11 | Hangul Jamo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Ethiopic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cherokee |  |  |  |  |  |  |  |  |  |  |
| 14 | Unified Canadian Aboriginal Syllabics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ogham |  |  |  | Runic |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  | Khmer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Mongolian |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 .. 1D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1E | Latin Extended Additional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1F | Greek Extended |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | General Punctuation |  |  |  |  |  |  |  |  |  | Super-/Subscripts |  |  |  |  |  |  |  |  | Currency Symbols |  |  |  |  |  |  |  | Comb. Mks. Symb. |  |
| 21 | Letterlike Symbols |  |  |  |  |  | Number Forms |  |  |  |  |  |  |  |  |  |  | Arrows |  |  |  |  |  |  |  |  |  |  |  |
| 22 | Mathematical Operators |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | Miscellaneous Technical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Control Pictures |  |  |  | O.C.R. |  |  | Enclosed Alphanumerics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 | Box Drawing |  |  |  |  |  |  |  |  |  |  |  |  |  | Block Elements |  |  |  |  |  |  | Geometric Shapes |  |  |  |  |  |  |  |
| 26 | Miscellaneous Symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | Dingbats |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | Braille Patterns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 .. 2D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2E |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CJK Radicals Supplement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2F | Kangxi Radicals |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ideographic Descr. |
| 30 | CJK Symbols And Punctuation |  |  |  |  | Hiragana |  |  |  |  |  |  |  |  |  |  |  |  |  | Katakana |  |  |  |  |  |  |  |  |  |
| 31 | Bopomofo |  |  | Hangul Compatibility Jamo |  |  |  |  |  |  |  |  |  |  |  | CJK Misc. |  |  |  |  | Bopomofo Ext. |  |  |  |  |  |  |  |  |
| 32 | Enclosed CJK Letters And Months |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | CJK Compatibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | = not graphic characters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | = reserved for future standardization |  |  |  |  |  |  |  |  |  |

NOTE:  Vertical boundaries within rows are indicated in approximate positions only.

Figure 4 - Overview of Rows 00 to 33 of the Basic Multilingual Plane